LOCAL CARTESIAN

10xxxxxx.1

# LEGAL RESTRICTIONS:

This Reusable Software Component (RSC) contains data with Unlimited Government Rights.

# DESCRIPTION:

LOCAL CARTESIAN is a C language code component that provides conversions between Geodetic coordinates (latitude, longitude and height) and Local Cartesian coordinates (X, Y and Z).

# CERTIFICATION LEVEL:

This RSC has been certified at level 4.  A level 4 component satisfies the criteria for reliability, testing, and documentation for the Army Reuse Center (ARC).  The component comes with test materials and a Reuse Manual that aids in integrating the component into a software system.

# LEVEL OF TESTING/ACCEPTANCE:

Unit and integration testing have been performed for the functions contained in this component.

# PURPOSE/INTENDED USE:

The purpose of LOCAL CARTESIAN is to provide a reusable component which supports the following coordinate conversions:

- Geodetic coordinates (latitude, longitude in radians and height in meters) to Local Cartesian coordinates (X, Y and Z in meters),

- Geocentric coordinates (u, v and w in meters) to Local Cartesian coordinates (X, Y and Z in meters),

- Local Cartesian coordinates (X, Y and Z in meters) to Geocentric coordinates (u, v and w in meters),

- Local Cartesian coordinates (X, Y and Z in meters) to Geodetic coordinates (latitude, longitude in radians and height in meters).

A particular ellipsoid is specified in terms of the following parameters:

- Semi-Major Axis (a):	Radius (in meters) at the equator, and

- Flattening (f):	Ratio of the difference between the semi-major axis and polar radius of the Earth to its semi-major axis.

A particular variation of Local Cartesian is specified in terms of the following parameters:

- Origin Longitude – Longitude (in radians) at the origin of the coordinate system,

- Origin Latitude – Latitude (in radians) at the origin of the coordinate system,

- Origin Height – Height (in meters) at the origin of the coordinate system,

- Orientation – Angle (in radians) which gives the direction of the Y axis relative to north.

# HARDWARE/ENVIRONMENT CONSTRAINTS:

There are no hardware or environment constraints.  There are no limitations.

# FUNCTIONS:

Set_Local_Cartesian_Parameters – This function sets the ellipsoid parameters and Local Cartesian parameters for the particular variation of Local Cartesian that is to be used in subsequent coordinate conversion operations.

Get_Local_Cartesian_Parameters – This function returns the current values of the ellipsoid parameters and Local Cartesian parameters.

Convert_Geocentric_To_Local_Cartesian – This function converts the specified geocentric coordinates (u, v and w in meters) to Local Cartesian coordinates (X, Y and Z in meters) using the current ellipsoid parameters and Local Cartesian parameters.

Convert_Geodetic_To_Local_Cartesian – This function converts the specified geodetic coordinates (latitude, longitude in radians and height in meters) to geocentric coordinates (U, V and W in meters) by calling the Convert_Geodetic_To_Geocentric function.  The geocentric coordinates are then converted to Local Cartesian coordinates (X, Y and Z in meters) using the current ellipsoid parameters and Local Cartesian parameters.

Convert_Local_Cartesian_To_Geocentric – This function converts the specified Local Cartesian coordinates (X, Y and Z in meters) to geocentric coordinates (u, v, and w in meters), using the current ellipsoid parameters and Local Cartesian parameters.

Convert_Local_Cartesian_To_Geodetic – This function converts the specified Local Cartesian coordinates (X, Y and Z in meters) to geocentric coordinates (U, V, and W in meters), using the current ellipsoid parameters and Local Cartesian parameters. The geocentric coordinates are then converted to geodetic coordinates (latitude, longitude in radians and height in meters) by calling the Convert_Geocentric_To_Geodetic function.

# EXAMPLE ApplicationS:

The following example illustrates how LOCAL CARTESIAN can be used to convert Geodetic coordinates to Local Cartesian coordinates and back again:

## Function Call:

status = Set_Local_Cartesian_Parameters (a, f,   Origin_Latitude,   Origin_Longitude,   Origin_Height,  Orientation)

## Inputs:

- a	6378137.0

- f	1 / 298.257223563

- Origin_Latitude	0.0

- Origin_Longitude	0.0

- Origin_Height	0.0

- Orientation                  0.0

## Function Call:

status = Convert_Geocentric_To_Local_Cartesian (u, v, w, X, Y, Z)

## Inputs:

- u:	0

- v:	50000

- w:	0

## Outputs:

- X:	50000

- Y:	21385

- Z:	-6367454

## Function Call:

status = Convert_Geodetic_To_Local_Cartesian (Latitude, Longitude, Height, X, Y, Z)

## Inputs:

- Latitude:	45.0

- Longitude:	-75.0

- Height:	700

## Outputs:

- X:	-4364136

- Y:	4487843

- Z:	-5208770

## Function Call:

status = Convert_Local_Cartesian_To_Geocentric (X, Y, Z, u, v, w)

## Inputs:

- X:	50000

- Y:	21385

- Z:	-6367454

## Outputs:

- u:	0

- v:	50000

- w:	0

## Function Call:

status = Convert_Local_Cartesian_To_Geodetic (X, Y, Z, Latitude, Longitude, Height)

## Inputs:

- X:	-4364136

- Y:	4487843

- Z:	-5208770

## Outputs:

- Latitude:	45.0

- Longitude:	-75.0

- Height:	700